Document for Console Mode  2019/12/18

1. FW Version of Bin File Check

- Function: get the fw version of bin file.

- Execute command-line: HUBIspTool.exe /FWB

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Search target Bin File according to the setting. Tool checked FW version of Bin File successfully, return code “0”.

If check failed on Hub 1, tool will set return code 1 (Set “Bit0” to “1”),

Hub 2, tool will set return code 2 (Set “Bit1” to “1”),

Hub 3, tool will set return code 4 (Set “Bit2” to “1”).

(Ex: Hub 2 and Hub 3 are failed, return code is 6(2+4).

Hub 1, Hub 2 and Hub 3 are failed, return code is 7(1+2+4))

- At last, save result to Version_BinFile.txt.

2. FW Version of Chip Check

- Function: get the fw version of hub.

- Execute command-line: HUBIspTool.exe /FWC

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Install filter driver of hub. If get file failed, tool will return code “-3”.

- Search target hub according to the setting again. If search no hub, tool will return code “-1”.

- Search target hub according to the setting. Tool got hub and checked FW version of hub successfully, return code “0”.

If check failed on Hub 1, tool will set return code 1 (Set “Bit0” to “1”),

Hub 2, tool will set return code 2 (Set “Bit1” to “1”),

Hub 3, tool will set return code 4 (Set “Bit2” to “1”).

(Ex: Hub 2 and Hub 3 are failed, return code is 6(2+4).

Hub 1, Hub 2 and Hub 3 are failed, return code is 7(1+2+4))

- And last, save result on Version_HubSPI. txt.

3. Hub Update FW

- Function: update FW to all hubs.

- Execute command-line: HUBIspTool.exe /DUD

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Search target hub according to the setting again. If search no hub, tool will return code “-1”.

- Update new FW to hub. Tool updated successfully, return code “0”.

If update failed on Hub 1, tool will set return code 1 (Set “Bit0” to “1”),

Hub 2, tool will set return code 2 (Set “Bit1” to “1”),

Hub 3, tool will set return code 4 (Set “Bit2” to “1”).

(Ex: Hub 2 and Hub 3 are failed, return code is 6(2+4).

Hub 1, Hub 2 and Hub 3 are failed, return code is 7(1+2+4))

- At last, save result to Result.txt

4. Install New Filter Driver

- Function: install new filter driver.

- Execute command-line: HUBIspTool.exe /AFD

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Search target hub according to the setting. If search no hub, tool will return code “-1”.

- Install filter driver to known hub. If tool executed failed, tool will return code “-3”.

- At last, save result to Result.txt

5. Uninstall New Filter Driver

- Function: uninstall new filter driver.

- Execute command-line: HUBIspTool /RFD

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Search target hub according to the setting. If search no hub, tool will return code “-1”.

- Uninstall filter driver to known hub. If tool executed failed, tool will return code “-3”.

- At last, save result to Result.txt

6. Hub Chip Reset FW

- Function: chip reset FW to all hubs.

- Execute command-line: HUBIspTool.exe /RST

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Search target hub according to the setting again. If search no hub, tool will return code “-1”.

- Execute 0.Function to hub. If tool executed failed, return code “-3”.

- At last, save result to Result.txt

7. Hub Write Protect Flash

- Function: write protect flash.

- Execute command-line: HUBIspTool.exe /WPT

- Check the setting of Setting.ini. If got file failed, tool will return code “0xFF”.

- Check the version of tool and API. If not matched, tool will return code “-2”.

- Search target hub according to the setting again. If search no hub, tool will return code “-1”.

- Execute 0.Function to hub. If tool executed failed, return code “-3”.

- At last, save result to Result.txt